- Digital Temperature Controller

Features Of Function

It is a mini-sized and Dintegrated intelligent  controller and applicable to the compressor of one Hp.

The main fuctions are :Temperature Display/Temperature Control/Manual,automatic defrost by turning off/illumination Control/Value Storing /Self Testing /Parameter Locking

Front panel operation

- Press“▼” and display “ON”, parameter unlock.

- Press ”SET” button and hold until display “E1”,and then press “▲”or “▼” to set lower limit of the temperature. Usually set E1=2 (When the temperature display 2 degree, the cabinet will stop working, of course the customer can set E1=3, or E1=4….)

- Press ”SET” button and hold until display “E2”,and then press “▲”or “▼” to set upper limit of the temperature. Usually set E2=6 (When the temperature display 6 degree, the cabinet will start working, of course the customer can set E2=7, E2=8…..)

- Press ”SET” button and hold until display “E3”, and then press “▲”or “▼” to set difference in temperature. Usually set E3=4 (E3=E2-E1)

- Press ”SET” button and hold until display “E4”, and then press “▲”or “▼” to set the delay time for compressor starting work. The scale would be 0-5 minutes. Usually set E4=3 (When the compressor stop working, if there is no any drink or water inside cooler, the air temperature inside the cooler will rise immediately, so we need to set 3 or 4, so that the compressor can start work delay 3 or 4 mintues to protect the compressor.

- Press ”SET” button and hold until display “E5”, and then press “▲”or “▼” to set the cold-room temperature. The scale would be from -5℃ to 5℃. (Customer are not allowed to re-set, factory should set E5 according to the temperature different.

- Press “SET” button and hold until display”E6”, set as “00”, no need to adjust it.

- Press ”SET” button and hold until display “F1”, and then press “▲”or “▼” to set defrost time. The scale would be 10-30 minutes. Usually set F1=25

- Press ”SET” button and hold until display “F2”, and then press “▲”or “▼” to set the interval of defrost. The scale would be 4-8 hours. Usually set F2=6

- Press ”SET” button and hold until display “F3”, and then press “▲”or “▼” to set the temperature of start working. The scale would be 10-20 ℃ Usually set F3=20

- Press “SET” button and hold until display “F4”,set as “00”, it is temperature display of defrost. No need to adjust.

- Press “SET” button and hold until display “C1”, it is temperature unit. Set “00”as Celsiur scale, set “1”as Fahrenheit.

- Press “¤”, light is on, press again, light is off.

- Refrigeration indicator light: The light is on when refrigeration, the light is off when the cold-room temperature is constant, the light flashed when delay start.

- Defrost indicator light: The light is on when defrosting, the light flashed when delay display defrost.

- Un-normal working mode:

·The control probe happened with  short circuit or high temperature (>45℃), it display “HH”, the control probe happened with open circuit and low temperature(<-45℃), it display “LL”, and the compressor start working 45 minutes and then stop working 15 minutes.

·The control probe happened with short circuit, open circuit or over limit, the refrigerator will top defrost, and will be under control of defrost time